Parameter Measurement for Speed and Torque Control of RC Servomotors on a Small-Size Humanoid Robot

Milton Ruas, Filipe M. Silva, Vítor M. Santos

a21824@alunos.det.ua.pt, fsilva@det.ua.pt, vsantos@mec.ua.pt

Department of Electronics and Telecommunications

Department of Mechanical Engineering

University of Aveiro, 3810-193 Aveiro, Portugal

# Introduction

In this paper, we describe parts of the control system architecture for a small-size 22 degrees of freedom (DOF) humanoid robot. The research focuses on the distributed control architecture, based on a CAN bus adopting a master/multi-slave arrangement, with a PIC microprocessor from Microchip on each unit. Emphasis is being placed on how actuators are driven to achieve an improved performance. Servomotors has been chosen for containing in one package built-in motor, gearbox and position controlling electronics, as extras of small dimensions and relatively low cost devices.

The servo is practical and robust because the control input is based on a digital signal – Pulse Width Modulation (PWM) – whose pulse width indicates the required position to be reached by the device. However, its controller isn’t aware of the motor load and there is no direct speed control, with a velocity dependent its load, which may be critical for trajectory planning algorithms. Additionally as the load increases a steady-state error occurs, turning the device into a highly non-linear actuator upon variable loads on the shaft.

The basic idea is to select measurable parameters that can be used to anticipate the influence of the disturbances on the process variables and use them to introduce software compensation actions via the closure of an outer position control loop. In this way it is possible to substitute hardware components without any further modifications, since all the control is made at the software’s level. In this work, procedures are described on how an external microcontroller can read the shaft position and current consumption in order to evaluate intrinsic velocity and torque developed by the motor. The experimental results obtained can be seen as a basis for further speed and torque control improvements.

# Experimental Setup

To evaluate the measurements required for the study of the servomotor advanced control, an entire system was set up; that includes a master and a slave unit controlling a servomotor properly fixed and loaded through a linkage of 10cm long. On the one hand, the master unit is connected to a computer through a RS-232 link, using MatLab software as the user’s interface. On the other hand, the slave unit is connected to the servo mechanism in two ways: by sending the desired servo position command and by reading the potentiometer feedback signal.

# Open-Loop Performance

After applying a step from, the first notorious observation is the presence of steady-sate errors with an 8º error for the larger load. Another observed anomaly is the unstable dynamic behaviour on position reading. The interesting part of this observation is that the motor shaft, physically, did not show this behaviour; a continuous and a fast motion to the final position were observed without speed inversions or oscillations.

In order to implement velocity control, a linear increase of position in successive steps was requested to the servo (ramp trajectory), whose time rate and amplitude variation imposes an average velocity. This way, speed contribution to current consumption will be diminished, depending mostly of the load torque, which was determinant to provide position reading stability. This dependence is related on how potentiometer signal is read. Different ground voltages between external measurement and servo’s internal controller produces a pulse addition, synchronized to PWM actuation signal, above the position voltage, whose length is proportional to current consumption. This way it is possible to measure current (pulse width) and position (minimum value) using the same output signal.

As Error: Reference source not found suggests, a planning algorithm is used to generate smooth trajectories that not violate the saturation limits and do not excite resonant modes of the system. The choice of a third-order polynomial function to generate the joint trajectory permitted us to satisfy joint positions and velocities continuity constraints with finite initial and final acceleration.

# Closed Loop Response

Introducing software compensation to enhance response performance an external controller, using an incremental PID algorithm, was added actuating directly on the PWM servo input using the shaft position and current consumption signal (provided by servo potentiometer output) as feedback.

Studying the effectiveness of the integral action (Error: Reference source not found), it can be observed that the steady state error is eliminated and the delay time is reduced (Error: Reference source not found). Increasing integral gain it can be observed an improved tracking accuracy, but at the expense of overshoot. Adding the proportional action (PI controller) Error: Reference source not found demonstrates that overshoot can be minimized maintaining the lag time. Nevertheless additional experiments showed that for high gains establishment time is deteriorated. For high demands in terms of time lag, transient response tends to be unstable with the presence of oscillations requiring the introduction of the derivative termto smooth transient response.

# Additional Improvements

The main drawback of the PID controller is that the load seen by the actuator can vary rapidly and substantially which makes it a highly non-linear system. At first sight, an improvement of the control performance could be achieved by using a torque control scheme adapting the control parameters accordingly to the torque applied. To this purpose, it is significant to estimate the electrical current which flows in the servomotor.

It was recognized that the time history of the estimated average current shows appreciable variation with the motor load. At this point, the potential of this measured parameter needs to be exploited. In view of the difficulties concerned with position-velocity control, the problem of torque control will deserve special attention.

# Conclusions

This paper described methods to perform position, velocity and current measurements in RC servomotors without hardware modifications. These parameters allow for more advanced motor control which is required for complex robots, such as the one in the current case: a 22-DOF humanoid. The results showed that velocity control and also trajectory planning is possible with such devices. Further, by having a process to evaluate average current, and relying also on velocity measurement, a door opens to perform some kind of torque control on these servomotors. This will allow highly robust control strategies of complex robots by means of simple RC servomotors. Therefore, issues like automatic gain, gain scheduling, continuous adaptation and variable structure control are being currently challenged.