Variable Speed Control of RC Servomotors for a Humanoid Robot

Milton Ruas, Filipe M. Silva, Vítor M. Santos

a23570@alunos.det.ua.pt, vsantos@mec.ua.pt, fsilva@det.ua.pt

Department of Electronics and Telecommunications

Department of Mechanical Engineering

University of Aveiro, 3810-193 Aveiro, Portugal

Abstract: no fim 

# Introduction

In recent years, the field of humanoid robotics has attracted the attention of a growing community, both from the industry and academia. It becomes increasingly evident the dichotomy in the styles used to design compact humanoid robots. On the one hand, several companies have unveiled walking robots with impressive designs and skills, as represented by Honda’s ASIMO [1] and Sony’s QRIO [2]. On the other hand, the continuous progress in robotics technology and the advances in computing hardware have promoted research on low-cost and easy-to-design humanoids, such as PINO [3], ESYS [4] and HanSaRam [4]. Here, the major challenge is to provide good performance of the control architecture and modularity at the system’s level.

In this paper, we describe parts of the control system architecture for a small-size 22 degrees of freedom (DOF) humanoid robot. The research focuses on the distributed control architecture, with the emphasis being placed on how actuators are driven to achieve a desired performance. For the dimensions involved, off-the-shelf actuation technologies do not offer significant alternatives other than small servomotors, such as those from FUTABA, HITEC and similar. There are several general characteristics that have made them actuators of choice in a large number of other applications: small, compact and relatively inexpensive. In fact, the servomotor itself has built-in motor, gearbox, position feedback mechanism and controlling electronics.

However, this common method of driving a robotic joint can deeply influence the system’s performance. First, it is well known that the control of the individual joints of the humanoid robot involves variation of the load inertia. Most certainly, such variations should be taken into account when trying to determine the proper control action; otherwise a decrease in performance will occur.

A second problem concerning the mentioned servomotors, is that they do not offer directly velocity control. Those servos can be controlled to move to any position just by using simple pulse width modulation (PWM). By design, servos drive to their commanded position fairly rapidly depending on the load (usually faster if the difference in position is larger).

A distributed set of microcontroller units is a key element to implement an adaptive scheme that compensates for the large changes in reflected inertia and providing variable velocity control.

Instead of changing the motor internals, as some other authors do, it was decided to improve its operation by software. That is achieved by a variable PWM throughout the full excursion of a joint. The algorithm is based on dynamic PWM tracking using the servo own potentiometer for feedback. In other words, the software tracks motor position with time and adjusts the PWM in order to accelerate or pause motor motion (the loop is closed back to the controller). Further, generating the control signals in each controller unit will reduce significantly the overhead on the controlling software.

# Project Overview and Framework

The main scope of the project beneath this paper has been the development of a humanoid platform to carry out research on control, navigation and perception, and also to offer opportunities for under and pos-graduate students to apply engineering methods and techniques in such ambitious and overwhelming endeavour. Purchasing a commercial platform carries prohibitive costs and it would reduce the involvement at the lowest levels of machine design, which was posed as a relevant pursuit for the desired engineering approach.

The ultimate goal of the project is to build a prototype capable of participating in the RoboCup humanoid league where a wide range of technologies need to be integrated and evaluated, giving added value for project-oriented education.

Model of the humanoid robot.

The most relevant achievements of this implementation include the distributed control architecture, based on a CAN bus, and the modularity at the system’s level.

## Distributed Control Approach

From the very beginning of the project, one major concern has been the development of a flexible control system to allow for short and possibly longer term developments. The key concept for the control architecture is the distributed approach, in which independent and self-contained tasks may allow a standalone operation. The platform was given a network of controllers connected by a CAN bus in a master-multi salve arrangement. Master and slave units are based on a PIC microcontroller.  shows a generic diagram of controlling units.

The master unit relays all slave units by dispatching medium and high level orders and by collecting sensorial data to be exchanged with the central unit. The central unit is currently an off-board computer but will be migrated to a local controller based on a PC104+ board with image processing capability.

Slaves can drive up to three servomotors, monitor their angular positions and electrical current consumption. The system joints have been grouped by vicinity criteria and are controlled by a dedicated board. Concerning additional sensors, each slave unit has the possibility of accepting a piggy-back board where additional circuit can lay to interface to other sensors (e.g., force-sensors, accelerometers and gyroscope).

- General architecture layout.

# Servomotors and Their Control

## Basics on RC servomotors

For several years now, hobbyists of radio-controlled (RC) models have been using compact motors in packages which consist of the actuator itself (motor), gears, and the unit responsible for position closed loop control; the unit includes a potentiometer and electronic circuits to perform that control.

This type of devices, so-called RC servomotors or, for short, servomotors or, so simply, servos, is so practical to use and robust, that their widespread use made them affordable to a large community. Currently, many brands of servos exist and roughly since the early 1990s some uniformity and standards have been established among brands for pins, sizes, and other technical issues [Fatlion, 2005].

The servo is practical and robust because the control input is based on a digital signal, whose pulse width indicates the required position to be reached by the device. The internal controller decodes this input pulse and tries to drive the motor up to the required position. However, the controller isn’t aware of the motor load and its velocity varies with the load; additionally, which may be critical, as the load increases a steady-state error occurs, turning the device into a highly non-linear actuator upon variable loads on the shaft.

In summary, the main problems faced when trying to use these, though small, powerful devices were: velocity control and conformity between input signal and output shaft position.

Fig.  1 shows one of the HITEC servomotors used in this project. Table 1 shows its main specifications according to some common information on flyers and datasheet spread over internet sites and vendors [HITEC, 2005]. It should be noted however that not all specs were actually verified as announced.

Fig.  1 - HITEC HS805BB servomotor

| Spec | Values |  |
| --- | --- | --- |
| Control system | Pulse Width Control 1.5 ms neutral |  |
| Voltage range | 4.8V to 6.0V |  |
| Teat voltage | @ 4.8V | @ 6.0V |
| Speed (no load) | 60º/0.19 s | 60º/0.14 s |
| Stall torque | 1.94 Nm | 2.42 Nm |
| Operating angle | 45º /one side pulse traveling 400s |  |
| Direction | clockwise/pulse traveling 1.5 to 1.9 ms |  |
| Current drain | 8mA (idle); 700mA (no load running) |  |
| Dead bandwidth | 8 s |  |
| Dimensions | 66 x 30 x 57.6 mm |  |
| Weight | 152g |  |

Table 1 - HITEC HS805BB servomotor specifications

Detailed information on how the internal controller works appears not to be well disseminated by the manufacturers, so most people working with servos rely on sparse or informal knowledge available in many websites [ePanorama, 2006].

## Experimental Setup

Now, we will describe the setup components and its most relevant aspects in order to understand the conditions present in the experimental procedures.

The main entity of the experimental setup is the servomotor since they will be used to control the robotic joints motion. RC model servos are fairly sophisticated devices that incorporate both position and speed feedback with a goal to provide precise position control. In normal use they compare the 1-2 ms, 50 hertz (50 Hz, 5-10% duty cycle) input pulse signal with an internal linear pulse generator controlled by the feedback servo position potentiometer (pot.) and by the motor back-EMF (voltage generated between power pulses) which is used as a speed sensor (Fig.3). The difference in pulse width, known as the error signal, is then “amplified” with a pulse stretcher. This circuit provides the servo control gain. The pulse stretcher output drives the servomotor through an H-bridge circuit to close the servo loop. The speed sensor feedback is normally used only to stabilize the position of the servo so the servo drives quickly to its commanded position with minimal overshoot. Fig.3 shows a Futaba S3003 circuit diagram along with the circuit functions believed to be in the BA6688. Although it is not the model used in our experimental setup (HITEC HS805BB) we believe that its main functioning aspects are the same due to its similar behaviour and product specifications.

An aspect that is worth to point is the PD controller type used to manipulate the servo motion. The proportional module uses the potentiometer error position to control the servo final position, and the speed sensor constitutes the derivative module in order to minimize overshoot. Due to the absence of the integrative component it is to expect a steady state error when moving high loads.

Fig. 1 - Futaba servomotor schematics.

In order to use the servo position feedback signal in a potential external controller, an extra output wire was connected to the potentiometer output whose signal will be used by a slave microcontroller unit to correct undesired deviations from the ideal response. In contrast with the Futaba S3003 servo, HITEC's potentiometer doesn't possess the same voltage reference (ground) as the servo itself, and so too, the microcontroller ADC converter used to measure its output voltage. Had to this, the potentiometer measured signal doesn't always correspond to servo position, introducing some offset deviations. By experimental observations we concluded that these deviations were related to the consumed current. As the H-bridge functioning uses an impulse input, its consumed current will follow the same behavior: a digital impulse using only a section of the PWM period. Because of that, potentiometer output, according to servo reference voltage, will present an offset deviation during current consumption, with the standard voltage (related to the servo position) in the remaining time (see Fig.5). An advantage can be taken from this behavior: as servo position can be measured from potentiometer output minimal voltage, current can also be known from the same output by its impulse length measure.

In the experimental procedures a mass load is applied indirectly to the servo shaft through an arm of 10 cm length recreating the torque appliances in the humanoid joints. To estimate the torque exerted, the servo was fixed in a mechanical lathe in which its neutral position (predefined to be the zero degrees position) corresponds to the maximum applied torque (gravity vector and link perpendicular between each other). On the other hand, the extreme locations (±90 degrees position) correspond to the null torque appliance. To test a wide range of torques, a set of weights were used, whose mass is indicated below:

- Mass 0: 0g (no load)

- Mass 1: 249g

- Mass 2: 454g;

- Mass 3: 714g

- Mass 4: 666g

Considering that the connection screw between the load and the arm link weights about 9g, the tested loads combines the weights above in the following way:

| Weights combination | Weights mass (g) | Effective load (+9g) | Maximum torq (N.m) |
| --- | --- | --- | --- |
| Mass 0 | 0 | 9 | 0.009 |
| Mass 1 | 249 | 258 | 0.253 |
| Mass 2 | 454 | 463 | 0.454 |
| Mass 4 | 666 | 675 | 0.662 |
| Mass 4+1 | 915 | 924 | 0.906 |
| Mass 4+2 | 1120 | 1129 | 1.108 |
| Mass 4+2+1 | 1369 | 1378 | 1.352 |

Table 2: Set of loads used during experiments.

Although the manufacturer indicates a maximum stall torque of 2.42 N.m at 6.0V voltage supply, in practice for loads higher than about 1.5 Kg (1.47 N.m), the servomotor was disabled to realize all traject between position extremes (±90 degrees). That is why the tested maximum load torque is only 1.352 N.m.

To recreate the same distributed architecture between microcontroller units in the humanoid robot, a master-slave system, connected themselves  through a CAN bus, was used (Fig.4), in which the slave unit is connected to the servo mechanism, using as output the servo control position signal and as input the potentiometer feedback signal, and the master unit is connected to a computer, through a RS-232 link, using MatLab software as interface. Fig.5 shows a photo of the experimental setup used to test the servomechanism.

Fig. 2 - Experimental setup diagram.

Ilustração dos blocos internos do servo

A ligação do potenciómetro ao exterior

A montagem para medição do binário

O esquema de ligação ao PIC e a interface de ADC

A lista de pesos usados

Etc...

## Open-Loop Performance

All control mention in this section reports to the application of a given pulse train with a specific width, therefore, the servo will is always presented with a Heavyside step in position. First, that is be done for a “large” step (equivalent to 90º) for several loads, and then smaller steps (fee degrees each) in order to simulate some kind of linear input and launching the basis for velocity control.

In all experiments reported in this section only the servo own controller will be responsible for the resulting performance. The only feedback information available, besides the visual input for humans, is the shaft position read at the servo’s potentiometer as described in the previous section; it shall be noted further that this process, too, requires care since apparently the internal controller interferes with the voltage drop on this potentiometer and that can affect external readings of the shaft position.

### Response to one large input step

After applying a step from -45º to +45º for different loads, the first notorious observation is the presence of steady-sate errors. For a low mass, the steady state error is practically negligible, but for the larger load (1129g) about 8º error remains after the transient phase (Fig.  2).

Fig.  2 - Step response for two loads from – 45º to +45º

Another observed anomaly in Fig.  2 is the unstable dynamic behavior on position reading, which shows at the beginning a sudden jump to a position below -45º and some oscillations during the path up to the final set point. The interesting part of this observation is that the motor shaft, physically, did not show this behaviour; a continuous and a fast motion to the final position were observed without speed inversions or oscillations. This tell us that, when carrying out long trajectories at the maximum speed (such as in response to large step inputs), the potentiometer output tends to become unstable, especially when increasing the load mass. This is related to the way the potentiometer voltage is read: the issue is related to different grounds for external measurement and internal controller. Therefore, the potentiometer output is consistent with the real servo position only when a very low current is being drained! For high loads (or fast motion) the servo increases the current demands and strangely adds an “extra” output pulse above the position voltage on the potentiometer. This occurs perfectly synchronized with the input PWM signal (Fig.  3).

Fig.  3 - Motor position at the potentiometer

The previous observations, as corroborated in Fig.  3, yield clues on the servomotor internal controller principles. After receiving each input pulse, the internal controller will provide current to the motor in the extent needed to accomplish the required angular displacement.

With the assumption that the internal controller uses a power bridge and internal PWM synthesis to push the motor, it is reasonable to accept that the “extra” pulse in the potentiometer level is due the fact that instantaneous power is applied during that period to the motor by the internal controller. It was also observed that the “extra” pulse on potentiometer level has a fixed top value. All things add up to conclude that the average power being transferred to the motor during a cycle of PWM (20 ms in these servos) is proportional to the width of this “extra” pulse, or in other words, admitting a fixed voltage applied by means of an internal bridge, the electric current being required by the motor is proportional to the width of the pulse, that we shall now onwards refer to as the “current pulse”.

Two main issues were concluded by these observations: care must be taken when reading the potentiometer value, and a means to measure electric current (in periods of 20 ms) was found! Only the accuracy in measuring the “current” pulse width will limit the accuracy in reading the current.

Strategies were then devised to measure motor position (i.e., potentiometer voltage). The simplest way is to sample the potentiometer several times, during the full period of one PWM pulse, and take the minimal value. Nevertheless, for high loads and high current demands, the current “pulse” can last the entire PWM free period, inhibiting position reading! Such a situation occurred in the experiments reported in Fig.  2: the low peaks in the transient behavior correspond to high peaks in voltage caused by the current pulse interference. This can be avoided by reading the potentiometer position exactly during the active part of the input PWM pulse. During this phase, no current generation occurs on the motor so reading can be done safely. However, other concerns arise regarding real time and PWM generation accuracy.

### Response to a “slope” input

In order to implement some sort of velocity control, some experiments were then carried out in a manner that a variable position would be successively requested to the servo. The rate at which each new position was imposed settled some kind of velocity. Nonetheless, the only way is still to give (smaller) position steps to the servo controller; only their magnitude and rate will dictate some desired “average velocity”.

This approach will generate an approximately linear increase (slope) for the position, which is to say, some constant velocity.

This way, the current demands will only practically depend on the load torque, because of the speed limitation introduced by the ramp input, and therefore the levels of current will be lower. In addition, beyond the position control, velocity control is introduced by the definition of the ramp length.

Fig.  4 - Response to a slope input for the highest load

In Fig.  4 it can be seen that, although the transient response has a very improved behavior, the steady state error still exists. Table 3 shows the results of an experiment carried out to stress this effect: the servo is requested to successively move a given weight to some positions; for each position, after motion completion, the potentiometer is sampled to obtain the real position where the servo is. Relating the positioning error with the static torque exerted in the joint,, where θ is the angular position, m the weight mass and g the gravity acceleration, a direct conclusion can be drawn: the higher the torque, the higher is the steady state error. This leads to the suspicion that the servo internal controller does not possess an integrator module which would bring the steady state error to zero, independently of the existing torque.

| Requested position º | measured position º | Error º | Torque (Nm) |
| --- | --- | --- | --- |
| -80 | -80 | 0 | 0.198 |
| -60 | -62 | 2 | 0.569 |
| -40 | -45 | 5 | 0.872 |
| -20 | -28 | 8 | 1.069 |
| 0 | -9 | 9 | 1.138 |
| +20 | +11 | 9 | 1.069 |
| +40 | +33 | 7 | 0.872 |
| +60 | +55 | 5 | 0.569 |
| +80 | +80 | 0 | 0.197 |

Table 3: Steady state error and physical torque exerted in the motor for a fixed set of positions using a 1138g load.

To correct these deviations, namely to reach the null steady state error and a low delay time in transient response, an external controller becomes necessary to improve these characteristics (Fig. ?).

: Servo controller diagram

Adaptive Controller

References

[Fatlion, 2005] The Gigantic Servo Chart, http://www.fatlion.com/sailplanes/servochart.html, seen in Feb 2006.

[HITEC, 2005] Analog Servos, http://www.hitecrc.de/store/home.php?cat=309, seen in Feb 2006

[ePanorama, 2006] Motor Controlling, http://documents.epanorama.net/links/motorcontrol.html, seen in Feb 2006.