Parameter Measurement for Speed and Torque Control of RC Servomotors on a Small-Size Humanoid Robot

Milton Ruas, Filipe M. Silva, Vítor M. Santos

a21824@alunos.det.ua.pt, fsilva@det.ua.pt, vsantos@mec.ua.pt

Department of Electronics and Telecommunications

Department of Mechanical Engineering

University of Aveiro, 3810-193 Aveiro, Portugal

Keywords: robotics, humanoid platform, control.

# Introduction

In this abstract, we describe parts of the control system architecture for a small-size 22 degrees of freedom humanoid robot. The research focuses on how actuators are driven to achieve an improved performance. Servomotors have been chosen for containing in one package built-in motor, gearbox and position controlling electronics and also for being compact and inexpensive. Servos are very robust because the control input is based on a digital signal (PWM), whose pulse width indicates the position to be reached. However, its controller isn’t aware of the output load and there is no direct speed control, which may be critical for trajectory planning algorithms. The basic idea is to measure output parameters to introduce software compensation via the closure of an outer position control loop. In this work, procedures are described on how an external microcontroller can read the servo outputs in order to enhance servo’s response.

# Main results

To implement velocity control, a ramp of positions in successive steps was requested (Fig.2), whose time rate and amplitude variation impose an average velocity. Another planning algorithm can be the generation of a third-order polynomial function to provide smooth trajectories that satisfies velocities continuity constraints.

After applying a ramp request, in open loop, one observation is the presence of steady-sate error and an appreciable time lag for a heavy load (Fig.2).

Table 1 - Requested position and measured position difference for a 1138g (static study).

| Requested position º | measured position º | Error º | Torque (Nm) |
| --- | --- | --- | --- |
| -60 | -62 | 2 | 0.569 |
| -40 | -45 | 5 | 0.872 |
| -20 | -28 | 8 | 1.069 |
| 0 | -9 | 9 | 1.138 |
| +20 | +11 | 9 | 1.069 |
| +40 | +33 | 7 | 0.872 |
| +60 | +55 | 5 | 0.569 |

Introducing software compensation, an incremental PID algorithm was added, actuating directly on the servo input using the shaft position and current consumption output as feedback (Fig.1). Studying the effectiveness of the PID controller it can be observed that the steady state error can be eliminated with no overshoot and an improved track accuracy (Fig.2, Fig.3, Fig.4).

Fig. 1 - Incremental PID algorithm.

Fig. 2 - Response to a ramp input for a 1139g load

Fig. 3 - Response to a slope input variating the proportional and integral gains.

Fig. 4 - Polynomial response for several proportional gains

The main drawback of the PID controller is that the load seen by the actuator can vary rapidly and substantially which makes it a highly non-linear system. An improvement of the control performance could be achieved by using a torque control scheme adapting the controller parameters accordingly to the torque applied. To this purpose, measurement of the electrical current which flows in the servomotor is done (Fig.6).

# Conclusions

The results showed that velocity control and also trajectory planning is possible with such devices. Further, by having a process to evaluate average current, and relying also on velocity measurement, a door opens to perform some kind of torque control on these servomotors. Therefore, issues like automatic gain, gain scheduling, continuous adaptation and variable structure control are being currently challenged.

Fig. 5 - Response for an optimized PID controller.

Fig. 6 - Current measurement: static case