As mentioned in chapter 2, the servomotor is composed by a DC motor and a PID controller that controls the shaft position accordingly to its input, providing a ramp response to the desired position with no oscillation and no steady state error when no load is applied to the shaft.

Applying a load, the controller response will be changed due to a modification in the system: adding a mass, more driving binary from the motor is required to move it.

Considerer a weight of mass m connected to the motor through a drag-link of length L. This system is similar to an inverted pendulum in which the joint is the motor, and the weight is on the other extremity of the link (Fig.X).

The dynamics is given by the equation below, in which c is the viscous friction coefficient of the joint, g is the gravity acceleration and θ is the opening angle between the link and a reference axis.

θ+cmL2θ+gLsinθ=0

As can be seen, this system has a non linear dynamics owing to the sin(θ) factor, making the use of the PID controller inappropriate to this situation. In some circumstances, the factor sin(θ) can be approximated to θ, making suitable the use of traditional controllers, nevertheless such approximation would limit θ variation to some degrees around a reference angle established a priori, what is not the case.

As a result, as far as the mass approximates the maximum binary point (link and gravity force vector perpendicular) the response delay time and steady state error tend to increase deteriorating the controller’s response. In the same way, these parameters are also related to the mass applied rising as far as we increase its weight.

To correct these deviations, a software controller was built following the model described below:

The desired position and the measured position (converted by the ADC) are compared and the result constitutes de error signal. To this signal is applied a proportional gain and a derivative compensation fundamental to improve the dynamic response. In addition, an integrative loop is used to clear the steady state error. Considering x(n) the desired position, y(n) the measured position and u(n) the servo input, the correctional compensator is given by:

un=un−1+KP⋅en+KD⋅en−en−1

We can observe that this equation represents a digital PID following the same nature of the standard servo controller. Although this second controller does not solve the non linearity problem, it offers the possibility to adapt the compensator to each specific scenario by direct modification of its parameters, something not possible with the standard controller.