# Product Information for Series ITV2000~3000

| ELECTRO-PNEUMATIC REGULATOR SERIES ITV |  |  |  |  |
| --- | --- | --- | --- | --- |
| Model |  | ITV201* | ITV203* | ITV205* |
|  |  | ITV301* | ITV303* | ITV305* |
| Minimum Supply Pressure |  | Set pressure +0.1MPa |  |  |
| Maximum Supply Pressure |  | 0.2MPa | 1.0MPa |  |
| Regulating Pressure Range |  | 0.005 to 0.1MPa | 0.005 to 0.5MPa | 0.005 to 0.9MPa |
| Power Supply | Voltage | 24V DC ± 10%, 12 to 15V DC |  |  |
|  | Current Consumption | Power supply voltage 24V DC type:0.12A or less Power supply voltage 12 to 15V DC type: 0.18A or less |  |  |
| Input Signal | Current Type¹ | 4 to 20 mA, 0 to 20mA |  |  |
|  | Voltage Type | 0 to 5V DC, 0 to 10V DC |  |  |
|  | Preset Input | 4 points |  |  |
| Input Impedance | Current Type¹ | 250 Ohms or less |  |  |
|  | Voltage Type | Approx. 6.5k Ohms |  |  |
|  | Preset Input | Approx. 2.7k Ohms |  |  |
| Output Signal² (Monitor Output) | Analog Output | 1 to 5V DC (load impedance: 1k Ohms or more) 4 to 20mA (sink type) (load impedance: 250 Ohms or less) |  |  |
|  | Switch Output | NPN open collector output: Max. 30V, 30mA PNP open collector output: Max. 30 mA |  |  |
| Linearity |  | Within ± 1% (full span) |  |  |
| Hysteresis |  | Within 0.5% (full span) |  |  |
| Repeatability |  | Within ± 0.5% (full span) |  |  |
| Sensitivity |  | Within 0.2% (full span) |  |  |
| Temperature characteristics |  | Within ± 0.12% (full span)/°C |  |  |
| Output Pressure Display | Accuracy | ± 3% (full span) |  |  |
|  | Minimum Unit | Mpa: 0.01, kgf/cm²: 0.01, bar: 0.01, PSI: 0.1³, kPA: 1 |  |  |
| Ambient and Fluid Temperature |  | 0 to 50°C (with no condensation) |  |  |
| Enclosure |  | IP65 equivalent |  |  |
| Weight |  | 350g |  |  |
| ¹) 2 wire type 4 to 20mA is not available. Power Supply voltage (24V DC or 12 to 15V DC) is required |  |  |  |  |
| ²) Select either analog output or switch output. Further, when switch ouput is selected, select either NPN output or PNP output |  |  |  |  |
| ³) The minimum unit for ITV205* is 1PSI |  |  |  |  |

Last Updated on 5/9/01 
By sf